Table 9.2  Installation Costs for a Heat Exchanger

(1984)

|  | Cost | Fraction of f.o.b. Equipment |
| --- | --- | --- |
| Direct project expenses |  |  |
| Direct materials |  |  |
| Equipment f.o.b. price, Cp | $10,000 | 1.0 CP |
| Materials used for installation, Cm = 0.71 Cp | 7,100 | .71 CP |
| Direct labor, CL = 0.37(Cp + Cm ) = 0.63 Cp | 6,300 | .63 CP |
| Total direct materials and labor | $23,400 | 2.34 CP |
| Indirect project expenses |  |  |
| Freight, insurance, taxes, CFIT = 0.08(Cp + Cm) | 1,400 | .14 CP |
| Construction overhead, CO = 0.70 CL | 4,400 | .44 CP |
| Contractor engineering expenses, CE = 0.15(Cp + CM) | 2,600 | .26 CP |
| Total indirect project costs | $ 8,400 | .84 CP |
| Bare-module capital, CBM | $31,800 | 3.18 CP |